Assessment Methodology Flowchart

Specify ultimate load factor (ULF) and/or working stress factor  (for working stress check) on vehicle

Does the Ultimate Strength Capacity  (Muc) exist?

N

Compare Mwc to HL effects (M) at normal, 10kph and 5kph speeds:

M = LL effect  x (1+DLA) x DF

Interpolate capacity  (See NOTE in the box below)

Perform HL comparison routine using the moment ratio method

Use specified DLA

Is a dynamic load allowance factor (DLA) specified in  the structure data file?  (For simply supported continuous bridges, each span may have a different DLA)

Check each structure on the route using the following procedure:

Use f=120/L for the frequency and find the max DLA from the BDS Code graph at ±0.1f [May customise Code graph  by editing the DLA-Frequency file using a text editor]

Does the “full overload” (e.g. 140% O/S) Working Stress Capacity (Mwc) exist?

Y

Y

N

Y

N

Compare Muc to HL effects (M) at normal, 10kph and 5kph speeds:

M = LL effect x ULF x (1+DLA) x DF

Does the structure have “Check working stress?” flag set to Yes?

Is the  working stress factor set to 140%?

Y

Y

FINISH

Does the base 100% working stress capacity exist?

Y

N

N

N

N

NOTE:  If working stress capacities are available for the 100% and 140% working stress levels, then intermediate stress level capacities will be linearly interpolated.   For example, the 125% stress level will be given by

0.625x (25/40)  of the difference between the 100% and 140% capacities.